PREVENTIVE MAINTENANCE OF AIR COMPRESSOR

| Equipment / Instrument Code No…………………………….. Location: …………………… Done Date……………… Next due date…………… |  |  |  |  |
| --- | --- | --- | --- | --- |
| Frequency for Preventive Maintenance : Three Months |  |  |  |  |
| Sr. No. | Check Points | Standard/ Range | Observation | Action (If Required) |
|  | Check all electrical connections in panel for loose and burnout | Should be Intact |  |  |
|  | Check return pipeline orifice for any blockage | Should be Clear |  |  |
|  | Check filters of dryer & change | Should be Clear |  |  |
|  | Check the Desiccants condition and change if required | Should be ok |  |  |
|  | Check moisture separator for any dust deposition | Should be Clean |  |  |
|  | Check oil separator element for change after every 4000 Hrs. | Should be Clear |  |  |
|  | Check motor for any uneven sound and lubricate with 20W40 oil & AP3 grade grease. | Should be Smooth without any noise |  |  |
| Entry made in the Log Book (if Applicable) :- (Yes / No)_____________ Conclusion: __________________________________________________________________________ _____________________________________________________________________________________ Technician Signature with Name: User H.O.D Comments: _________________________________________________________________ Maintenance Head Comments (If Any)____________________________________________________ USER HOD SIGNATURE………………….. MAINTENANCE HOD SIGNATURE…………… |  |  |  |  |

Preventive maintenance Checklist No.: KHPL/PMT/004	Revision No. : R-00