| Roles | Name | Designation | Signature | Date |
| --- | --- | --- | --- | --- |
| Author |  | HR Manager |  |  |
| Reviewer |  | Management Representative |  |  |
| Approval |  | Director |  |  |

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| DCO no. | Revision | Change Description | Author | Effective Date |
| DCO 2022-001 | 01 | Initial Release |  | 01 Oct 2022 |

- Gowning Room Cleaning Procedure : Shoe and head covers are required in this area. Begin all cleaning at the top and finish at the bottom of any equipment or surface to be cleaned. 1. Empty trash containers and replace plastic liners. 2. Replenish stock of shoe covers, masks, head covers, gowns, gloves and put into proper areas. 3. Fill dispenser with 0.45 u filtered 70% Isopropyl alcohol. 4. Mix cleaning solution with water 5. Fill a stainless-steel basin with cleaning solution and begin wiping in this order: the boot box; stool; wash station; top and outsides of all cabinets; counter tops; and the tops of the trash containers. 6. Remove excess debris and wet mop the floor with cleaning solution. 7. Fill sprayer with cleaning solution and wet the floor; allow it to air dry. 8. Gown according to gowning procedures and go into the filling area

Weekly cleaning procedure: All daily cleaning requirements are included in the weekly cleaning requirements. The additions to the weekly cleaning are ceilings, walls, and the internal work surfaces of the laminar-flow work benches. 1. Use only one entrance and one exit when cleaning. The cleaning direction will flow from entrance to exit. 2. Remove all non-essential equipment from the room and clean it. 3. Begin cleaning the room by wiping the entire ceiling area with the cleaning solution.

Frequent changes of the cleaning solution and the wiping cloths are needed for effective cleaning of large areas such as the ceilings and walls. Make new solution and change wiping cloths when dirty. - Clean air vents, lighting fixtures, and sprinkler heads. - Clean the walls next, starting at the top and cleaning toward the floor. - When the ceilings and walls are cleaned, follow steps of the Daily Cleaning Requirements for the gowning room. - Remove debris and wet mop the floor with cleaning solution. - Fill the sprayer with cleaning solution and wet the floor. Allow to air dry. - Gown according to procedure and go into the Filling room

Procedure:

Entry procedure

- Before entering into the working area, remove the footwear and keep it allocated rack lying in the change room and wear the lab footwear

- Put on the washed apron

- Deposit the belongings in the respective locker (remove jewellery, makeup, heavy coats, sweatshirts, and other "extra" items)

- Before entering the workplace, wash the hands.

- Put on a cap and to make sure all hair is covered.

- Select, inspect, clean, and then put on safety glasses or goggles.

- Put on the gloves and shoe cover before entering

- Use hand sanitizer or another cleaning agent.

- Enter the air shower and use it until it turns off, making sure to turn so the air can get to every spot.

- Enter cleanroom.

De-gowning Steps

- Remove boot or shoe covers. If they are to be used again, store them in the designated container or bag.

- If you're wearing two sets of gloves, discard the outer pair. Otherwise, remove gloves last.

- De-gown by removing the coverall gown. Hang in the approved area or discard.

- Remove eyewear and place it in the proper storage container.

- Remove hood and place in an approved area, or discard.

- Exit the gowning room.

- Remove and discard the face mask and cap .

- Remove and discard the pair of gloves.

- Place disposable items and recyclable items into the appropriate receptacles.

Exit procedure

- Before leaving the working area, remove the footwear and keep it allocated rack lying in the change room and wear the lab footwear

- Withdraw the belongings in the respective locker

- Before leaving the workplace, wash the hands.

Images for gowning and De gowning